- A review of the geodesy calculations in ROPP

- Ian Culverwell

- Met Office, UK

7th February 2012

1. Introduction

A review of the geodesy calculations in ROPP was suggested by the ROPP2.0 beta reviewer (GRAS SAF 2008). His concerns were that these calculations are not being done sufficiently accurately and/or consistently in ROPP, and he suggested that the treatment of geodetic terms should be re-examined.

There seem to be two aspects of the reviewer’s concern: insufficient accuracy in the transformation between geometric and geopotential height, and the apparent inconsistency between the effective radius of the earth and the radius of curvature of the earth at the tangent point.

Lewis (GRAS-SAF Report 02, hereafter GSR-02, 2007) compared the early ROPP implementations of gravity, effective radius and geopotential, based on expressions in the Smithsonian Meteorological Tables (SMT), and versions which have since been implemented in ROPP, based on Mahoney’s 2008a calculations using Somigliana’s equation.  He found that the fractional differences between gravity and geopotential were negligible (<10-5), but that the fractional difference in effective radius of ~5x10-5 equated to ~350m, which was considered excessive. This is why ROPP has consistently used geodesy routines based on Somigliana’s equation since ROPP1.2.

A later follow-up paper (Lewis, pers. comm., 2010) discussed the differences between these later expressions for gravity, effective radius and geopotential and those used in the Invert package, which is used to process GRAS-SAF NRT products. He found various inconsistencies between the two sets of expressions: a 3x10-5 fractional difference in the equatorial gravity, and up to 300m difference in effective radius. The differences in geopotential height were generally less than 1m, however, and therefore considered negligible.

The treatment of the radius of curvature in ROPP has developed since ROPP2.0 was reviewed, and, as the reviewer recommended, expressions involving the actual radius of curvature of the section of the ellipsoid intersected by the occultation plane are now used in the ropp_utils/coordinates routines that are used by the ropp_pp module of ROPP.

This report is an attempt to tie all these threads together, and give a coherent account of the current geodesy calculations in ROPP.

Geodesy – the study of the measurement and representation of the Earth, including its gravitational field – is important to radio occultation in two distinct ways: “physically”, in connection with the effects of gravity on the atmosphere; and “mathematically”, in connection with the natural co-ordinate system in which to discuss the bending of radio signals during an occultation. These two facets will be discussed separately, before describing how they come together.

2. Normal gravity

2.1 Variation with latitude on surface of earth

Numerous expressions for the normal gravity on the geoid appear in the literature.

The most authoritative reference appears to be the International Gravity Formula (Lambert 1945). Writing g(h, φ) for the effective (ie including rotational effects) gravity at geometric height h and geographic (or geodetic) latitude φ, Lambert gives the surface gravity as

g(0, φ) = ge [1 + βsin2 φ – β1sin22φ]                                                (1)

where (ignoring 3rd order terms and some geometric shape factors like everyone else)

β = (5/2)m – f – (17/14)mf  ≈ 5.3x10-3                                            (2)

and

β1 = (f/8)(f + 2β) = (f/8)(5m – f) ≈ 5.9x10-6                                          (3)

Here, ge = g(0, 0) = surface gravity at equator, f = (a – b)/a is the flattening of the assumed oblate ellipsoid of the mean sea level geoid with semi-major and semi-minor axes a and b respectively, and m = ω2a/g(0,0) is the ratio of centrifugal to gravitational force on the equator. For the earth, not coincidentally, f and m are numerically similar: f ≈ m ≈ 1/300. This makes      β > 0, which implies that g increases towards the poles, as might be expected from the reduced centrifugal “dilution” of gravity and the smaller distance to the earth’s centre.

Equation (1), or something very much like it, appears throughout the literature (eg Smithsonian Meteorological Tables 1896, Mahoney 2008a).

Somigliana’s equation for the surface gravity, as currently used in ROPP (ropp_utils/geodesy/gravity.f90), takes the closed form (Heiskanen and Moritz, 1967):

g(0, φ) = ge [1 + kssin2φ] / √[1 – e2sin2φ]                                             (4)

where

ks (Somigliana’s constant) = (bgp – age) / age  ≈ 1.9x10-3                              (5)

and

e (eccentricity) = √[(a2 – b2)/a2]  ≈ 8.2x10-2                                        (6)

(gp = g(0, π/2) = surface gravity at pole). Li and Götze (2001) then quote the following second order expansion of eqn (4):

g(0, φ) = ge [1 + f* sin2φ – f4 sin22φ]                                                (7)

where

f* = (gp – ge )/ge  ≈ 5.3x10-3                                                       (8)

and

f4 = (f/2) (5m – f)  ≈ 4 x 5.9x10-6.                                                 (9)

(Eqn (9) is a bit deus ex machina since it requires something like eqn (16) below to get the normalised rotation rate m into it. More consistent would be to say:

f* = (1 + ks) (1+ f)  ≈ 5.3x10-3                                                       (8’)

and

f4 = f (3f/2 + ks)    ≈ 4 x 5.9x10-6,                                                   (9’)

).

Note that ks (or gp) in Somigliana’s eqn (4) must be specified directly: it is not derived from more elementary terms. Lambert quotes a closed form expression for g(0, φ), due to Pizetti  (1894), which has the same form as Somigliana’s equation, but with a derived value of (effectively) ks:

g(0, φ) = ge [1 + (mC – e2)sin2φ] / √[1 – e2sin2φ]                              (10)

where

C = 5/2 – 13/7 ε2   ≈  2.5                                                           (11)

and

ε (second eccentricity) = √[(a2 – b2)/b2]  ≈  8.2x10-3                            (12)

ROPP versions before 1.2 used an expression based on the Smithsonian Meteorological Tables (Lewis 2007, from a forerunner of Mahoney 2008a), which was a particular representation of Lambert’s equation:

g(0, φ) = ge [1 + βsin2φ  – β1sin22φ]                                                (13)

in which

ge = 9.780456 ms-2; β = 5.2885x10-3; β1 = 5.9x10-6                                 (14)

Finally, the surface gravity used in the Invert code is given by (Lewis, 2010):

g(0, φ) = ge [1 + f2 sin2φ – ¼ f4 sin22φ]                                              (15)

where

ge = GM / Re2 (1 – f + 3m/2 – 15mf/14)

f2 = –f + 5m/2 – 17mf/14 + 15m2/4

f4 = –f2/2 + 5mf/2

This expression doesn’t quite follow from Lambert’s (1) or Pizetti’s (10) formulas, but it does agree to first order in m and f. In fact, all three expressions imply that, to first order in small quantities, the ratio of the polar to the equatorial surface gravity is:

gp / ge = g(0, π/2) / g(0, 0) = 1 +  5/2 m – f,                                            (16)

a famous result due originally to Clairaut (White et al, 2008). Since f ≈ m ≈ 1/300, this means that surface gravity is about 0.5% bigger at the poles than at the equator.

Equation (4) is currently used in ropp_utils/geodesy/gravity.f90 with the parameters

ge = 9.7803253359 (ms-2), ks = 1.931853e-3, e = 0.081819.

Figure 1 compares expressions (1), (4), (7), (10), (13) and (15) for the surface gravity as a function of latitude. Lambert, Somigliana, Li and Gotze and Pizetti are effectively identical. The largest difference is between Invert and Somigliana, but even this incurs a fractional difference of less than 30 parts per million, which is probably negligible (see Sec 4).

Conclusion: No need to amend the surface gravity expression in ROPP.

Fig 1: Latitudinal variation of surface gravity defined by Somigliana’s equation, as used in ROPP, and fractional differences from it of alternative expressions – see text for details.

2.2 Variation with altitude

Lambert 1948 also quotes the International Gravity Formula expression for the variation of gravity with altitude normal to the ellipsoid. Since this does not seem to be generally available, we quote it in full:

g(h, φ) = g(0, φ) –

2ge(h/a) [1 + α0 + α1cos(2φ) + α2 cos(4φ)]  +

3ge(h/a)2 [1 + α3 + α4cos(2φ)]                                          (17)

where, in the notation of Sec 2.1,

α0 = (9/4)m –(1/2)f + f2 – (173/112)mf  ≈ 6.1x10-3

α1  = (3/2)f – (5/4)m + (1/4)f2 + (13/7)mf  ≈ 7.3x10-4

α2  = (1/4)f2 – (5/16)mf  ≈ –8.2x10-7                                                  (18)

α3  =  – (2/3)f + (7/3)m  ≈ 5.9x10-3

α4  = 3f – (5/2)m  ≈ 1.4x10-3

and a is the semi-major axis of the ellipsoid of the earth.  This expression appears to derive from a complicated expansion of the geopotential in ellipsoidal co-ordinates – see, for example, Mahoney 2008b.

If m = f = 0 (non-rotating, spherical planet), all the α’s vanish and eqn (17) reduces to the first two terms in the expansion of

g(h, φ) = g(0, φ) [a / (a + h)]2                                                         (19)

as would be expected from the inverse square law of gravity.

3. Effective radius of earth

In ROPP (and elsewhere) it is convenient to express the variation of gravity with latitude and height in an analogous way to eqn (19), by means of a fictitious, latitudinally varying, effective radius Reff(φ), thus:

g(h, φ) = g(0, φ) [Reff (φ) / (Reff(φ) + h)]2                                       (20)

[Aside: there is no reason in general why g should fall off with height along a given latitude line as ~ 1/(R + h)2. It isn’t true, for instance, for a uniform non-rotating rod lying between (-1,0) and (1,0) in the x-z plane. The points (√2, 0) and (0, 1) lie on an equipotential surface (an ellipse, in fact). In the x-direction, g(√2+h, 0) = 2/((√2+h)2 – 1); in the y-direction; g(1+h, π/2) = 2/(1+h)/ √(1+(1+h)2). Neither of these has the form of eqn (20), except in the limits of very small and (of course) very large h. Interestingly, these limits have different Reffs – at both latitudes.]

Assuming eqn (20) is true, then Reff(φ) is defined by:

Reff(φ) =  –2 g(0, φ) / gh(0, φ)                                                   (21)

Note that the effective radius is a purely theoretical construct, invented to encode gh at the surface by means of eqn (21), so that g(h, φ) can be written as in eqn (20).  It bears no relation to the actual radius of the earth.  This becomes clear when eqns (1) and (17) are substituted into eqn (21): we obtain, to first order,

Reff(φ) / a = (1 + βsin2 φ) / (1 + α0 + α1 – 2 α1sin2 φ)                              (22)

Noting, as Mahoney 2008a does, that β and α1 are positive, this means that Reff(φ) increases towards the poles. (The equator-to-pole reduction in gh= ∂g/∂h in the denominator is in fact about 1/3 that of the increase of g in the numerator.) The fictitious nature of the effective radius is therefore clear – although perhaps not as well known as it should be.

[Aside: in the uniform rod example above, Reff(0) = 1/√2 and Reff(π/2) = 4/3. As in eqn (22), it therefore increases poleward, unlike the radius of the geoid which decreases from √2 at the equator to 1 at the pole.]

Mahoney quotes a corresponding expression to eqn (11) for Somigliana’s equation for gravity, eqn (4). By now, the reader will not be surprised to learn that I have been unable to reproduce this derivation. But the resulting effective radius, apparently given by

Reff(φ) / a = 1 / (1 + f + m – 2 f sin2 φ)                                              (23)

in which a = 6378137.0 (m), f = 0.003352811 and m = 0.003449787, is the one currently implemented in ropp_utils/geodesy/r_eff.f90.

ROPP versions before 1.2 used an expression for effective radius based on the Smithsonian Meteorological Tables (Lewis 2007, from a forerunner of Mahoney 2008a):

Reff(φ) / a = [1 + βsin2 φ  – β1sin22φ] / (α0 + α1 cos2φ + α2 cos4φ)               (24)

where β and β1 are given by eqn (14), and

α0 = 3.085462x10-6, α1 = 2.27x10-9 and α2 = –2.0x10-12                           (25)

For completeness, the effective radius used in the Invert code is given by (Lewis, 2010):

Reff(φ) / a = gsfc/ge / (1 + f + m + (–3f + 5m/2)sin2φ)                                (26)

where gsfc/ge is given by eqn (15). To first order in m and f, eqn (25) agrees with eqn (23). Putting f = m = ½e2, both yield, to first order in e2,

Reff(φ) / a ≈ 1 – e2 cos2φ                                                       (27)

Figure 2 compares expressions (22), (23), (24) and (26).  For comic relief, it also shows the actual radius of the ellipsoid as a function of latitude.  The largest fractional difference in effective radii, between the expressions based on Somigliana’s equation (eqn 23) and the Smithsonian Tables (eqn 24) is about 80 parts per million. As we will shortly see, this has a negligible impact in ROPP applications.

Conclusion: No need to amend the effective radius expression in ROPP.

Fig 2: Latitudinal variation of effective radius defined by Somigliana’s equation, as used in ROPP, and fractional differences from it of alternative expressions – see text for details.  Actual radius of earth also shown.

4. Geopotential height

The key thing for radio occultation is the geopotential height Z at geometric height h, which is the (normalised) energy required to lift a unit mass air parcel from the geoid to that height:

Z(h, φ) = gref-1 ∫0h g(h', φ) dh'                                                   (28)

where gref  is a normalising acceleration due to gravity.

Assuming an inverse square law of gravity at each latitude, as in eqn (20), eqn (28) gives

Z(h, φ) = (g(0, φ)/gref) Reff(φ) h / (Reff(φ) + h)                              (29)

Since Reff (φ) >> h, this implies

Z(h, φ) ≈ (g(0, φ)/gref) h                                                        (30)

so that Z(h, φ) is almost independent of Reff(φ). More precisely, eqn (29) shows that (dropping the arguments for clarity)

δZ/Z = δg/g  + h/(Reff + h)  δReff/Reff                                      (31)

In ROPP work, h <~ 60km, so h/(Reff + h) <~ 10-2, which means, as hinted above, that a fractional difference in effective radius has a far smaller effect on geopotential height than a corresponding change in surface gravity.

Fig 1 shows that |δg/g| < 30x10-6, and Fig 2 shows that |δReff/Reff | < 80x10-6, Overall, then, the fractional difference in geopotential height is dominated by the difference in surface gravities. This is borne out by Figure 3, which shows the differences in geopotential heights (eqn (29)) calculated using formulas for g and Reff calculated using Somigliana’s formulas, Lambert’s formulas, the Invert code and the SMT85 tables. It is clear that δZ/Z in Fig 3 largely reflects δg/g in Fig 1. The largest difference is between the Invert code and Somigliana’s expression, as would be expected from Fig 1 and eqn 31, but even this is only 30 parts per million, which equates to 2.4m at 80km, which is negligible. (It equates to a temperature difference of about 0.012K at a typical mesospheric lapse rate of 5K/km. In the lower troposphere, at around 2km, a δZ/Z of 30x10-6 equates to 0.06m, which corresponds to a specific humidity difference of 0.00015g/kg at a typical hydrolapse rate of 2.5g/kg/km.)

Similarly, for the inverse relation between geometric height and geopotential height, also needed in ROPP,

h(Z, φ) =  Reff(φ) Z / ((g(0, φ)/gref) Reff(φ) – Z)                               (32)

we find approximately

δh/h = – δg/g  – (h/Reff) δReff /Reff.                                        (33)

Since h << Reff, the same weak dependence on Reff(φ) results.

[Aside: Lewis 2007 found changes in Z <~ 1m between the earlier SMT expressions for g and Reff, and the later ones based on Somigliana’s eqn, when the difference in Reff was <~ 300m. As above, is clear that in this case the majority of the difference was coming from the term in δg /g.]

Conclusion: No need to amend the surface gravity or effective radius expressions in ROPP.

Fig 3: Latitudinal and vertical variation of geopotential height defined by Somigliana’s equation, as used in ROPP, and fractional differences from it of alternative expressions – see text for details.

5. Radius of curvature

Sections 2, 3 and 4 have been associated with the mass and rotation of the earth, and their influence on its gravitational field. The radius of curvature is a more geometrical idea, associated (in the context of radio occultation) with the curvature of the earth in the vicinity of the tangent point. The local centre of curvature, thus defined, is the natural place to site the origin of the co-ordinate system used to calculate the occultation path, bending angle, etc.

In this co-ordinate system, the local shape of the geoid is approximated by a circle with the same curvature as the intersection of the (assumed) ellipsoidal mean sea level geoid with the plane of the occultation (Rodgers 2000, p 148-149). The resulting radius of curvature is very different from the actual local radius of the earth: greater at the pole, and (except for strictly E-W occultations) smaller at the equator. Rodgers quotes the formulas:

RNS(φ) = a2b2(a2cos2φ + b2sin2φ)-3/2                                                 (34)

REW(φ) = a2(a2cos2φ + b2sin2φ)-1/2                                                    (35)

Rc(φ, α)-1 = RNS(φ)-1 cos2α + REW(φ)-1 sin2α                                        (36)

in which RNS is the “meridional” radius of curvature at latitude φ, REW is the “normal” radius of curvature at latitude φ and Rc is the radius of curvature in the place at azimuth (ie bearing of tangent plane with respect to true north) α.

In ROPP, eqn (36) is evaluated in ropp_utils/ coordinates/curvature.f90.

The (arithmetic) average of Rc, assuming all azimuths α are equally likely, turns out to be:

<Rc>(φ) = √(RNS (φ) REW (φ)),                                                   (37)

which gives a simple estimate of the latitudinal variation of Rc(φ).

[Aside: for the record, the harmonic average over α of Rc(φ, α) equals 2 RNS REW / (RNS+REW), which is almost identical to eqn (37) numerically, because of the small oblateness of the earth.]

For small e, <Rc>(φ) ≈ a(1 – ½e2cos2φ) (however it is averaged), where a is the semi-major axis of the ellipsoid and e = √(1 – (b/a)2) ≈ 0.08 is the ellipticity. ½ae2 ≈ 21km, the “equatorial excess radius”, a – b.  We therefore have

<Rc>(φ) / a ≈ 1  – e2 cos2φ  + ½e2                                                 (38)

Comparison with eqn (27) shows that, excitingly, the mean radius of curvature exceeds the effective radius of the earth by a constant offset of about 21km.

RNS , REW and <Rc>(φ) are plotted as a function of latitude in Fig 4.  The actual radius of the earth as a function of latitude, given to first order in e2 by

Rearth(φ) / a ≈ 1 –  ½e2 sin2φ,                                                   (39)

is also plotted.  Clearly these last two expressions do not match, which means that the (mean) centre of curvature is displaced from the centre of the earth by an amount varying from 21km at the equator to 42km at the pole, and whose magnitude (≈ ½ae2 √(1 – 2sin2φ + 5sin4φ)) is also plotted in Fig 4.  Correctly accounting for the oblateness of the earth in this way is vital for the correct interpretation of radio occultation measurements (Kursinski et al 2000).

Conclusion: No reason to amend the curvature calculations in ROPP.

Fig 4: Latitudinal variation of various radii of curvature, and of distance between centre of curvature and centre of earth.

6. Undulation

Up to know it has been assumed that the geoid – that is, the equipotential surface that best fits the earth’s mean sea level – is described as an ellipsoid: usually (and in ROPP) the WGS84 ellipsoid (NGA 2010).  This is not quite true: the two differ because of localised variations in the earth’s gravitational field.  The difference between the height of the geoid and the height of the ellipsoid is known as the undulation, u.

The undulation of the EGM96 geoid model (NASA 2004) is plotted in Fig 5a, and the magnitude of its gradient is plotted in Fig 5b (note the log colour scale). The range of the undulation is no more than about ±100m, and appears relatively smooth. The highest gradients follow mountain ranges and undersea ridges.  About half the points have undulation gradients greater than about 2e-5 m/m in magnitude. Over a 400km drift in the tangent point during an occultation, this could accumulate to a change in undulation of about 8m, which could be significant. Presently in ROPP, however, a fixed undulation is assumed throughout the occultation.

In most ROPP applications the undulation is provided by the data provider as a single number. In the ropp_pp module, which generates bending angles and refractivities from level1a excess phase data, the (single) undulation at the nominal tangent point location is calculated from the (order and degree 360) spherical harmonic expansion of the EGM96 geoid provided on the website (NASA 2004). The highest order spherical harmonic in this expansion has a wavelength of 1º ~ 110km along the equator, which should be enough to resolve undulations on these scales. If, however, a more highly resolved undulation were required at some point in the future, then it may become necessary to use a higher order expansion, such as EGM2008 (NGA 2010), which is of order and degree 2159, and which has a minimum wavelength of about 18km.

[Aside: In ropp_pp, why not simply store the 2D field shown in Fig 5a and interpolate from that?]

The undulation is needed to convert between impact parameters n(r)r, which involve distances from the centre of curvature Rc (Sec 5), and geometric heights h, which are measured with respect to the geoid:

r = h + u + Rc                                                              (40)

The geometric heights h are converted to and from geopotential heights Z, on which model variables are defined, by means of eqns (29) and (32). These equations, in which gref  = 9.80665 ms-2, are the ones currently implemented in ropp_utils/geodesy/geometric2geopotential.f90 and ropp_utils/geodesy/geopotential2geometric.f90 respectively.

In this sense the undulation provides a connection between the physical and the mathematical aspects of geodesy in radio occultation: the geoid and the ellipsoid.

Conclusion: Unless undulations that vary through the occultation are needed, there is no need to improve the treatment of undulation in ROPP.

Fig 5: Spatial variation of: (top) undulation (Fig 5a); and (bottom) magnitude of gradient of undulation (Fig 5b). From EGM96 (NASA 2004).

7. Response to ROPP2.0 beta reviewer’s comments

The ROPP2.0 beta reviewer (GRAS SAF 2008) made two specific comments on the geodesy calculations in ROPP.

Geometric vs geopotential transformations: Several details were found not to be of sufficient accuracy. The expressions used conform to estimations that are standard in meteorological contexts. However, they are less accurate than the GPSRO technology is, and thus will become the limiting factor for the data. The reviewer considers this as “correct” in the sense that conforms to standard practice. However this should be addressed in future ROPP versions.

No further details are available.  The beta review of ROPP2.0 took place during August/September 2008. The last change to the gravity and effective radius calculations in ROPP (from expressions based on SMT tables to those based on Somigliana’s equation, as recommended by Lewis (2007)), were made for ROPP1.2 in June 2008.  Therefore the reviewer’s objections – whatever they were – remain. However, he admits that this is standard practice. And Figures 1-3, and the discussion around them, suggests that any infelicities in this regard are likely to have minor impact in ROPP.

Earth radius: The quantity estimated (Somigliana) is not the radius of curvature. It is not an accurate estimation of the radius of curvature (which depends not only on latitude, but also on the azimuth). It is not more accurate for the estimation of the geopotential than a fixed average radius. Instead, it is much more computationally expensive. The error is comparatively small in the way and context that this quantity is used in this release. However, it could lead to very substantial errors if this quantity was used in other contexts within GPSRO applications where the Earth radius is also needed. This should be addressed as future releases of ROPP may very likely require this quantity for other purposes.

The distinction between effective radius and radius of curvature has been stressed above. There appears to be no confusion in ROPP2.0 between the two: where needed, the radius of curvature is read from input files. (The ropp_pp module was very rudimentary at ROPP2.0, and contained none of the routines that, in later releases, calculate the radius of curvature as in eqn (36).)  We agree with the reviewer that a varying Reff is not much more accurate for calculations of geopotential than a constant one.  A ballpark difference between them of 20km (see Fig 2) would, according to eqn (31), result in a fractional difference in geopotential height of about 30 parts per million at 60km.  Coincidentally, this matches the contribution to δZ/Z arising from the maximum uncertainty in δg/g shown in Fig 1, as discussed (and dismissed) in Sec 3. For the same reasons, then, a constant effective radius would probably be sufficiently accurate for ROPP.  Then again, a latitudinally varying effective radius is not more inaccurate.  We dispute that using eqn (23) is “much more expensive” than using a constant Reff: both take a negligible fraction of the total CPU time of any ROPP calculation. In view of the pain of undoing the calculation of effective radius, we feel it is acceptable to leave it alone.

We think the reviewer’s last two sentences refer to the risks of mistaking Reff for Rc. We agree that this could potentially be a problem, but as pointed out above, the two quantities are now calculated independently and correctly in ROPP.

Conclusion: In the absence of further information as to the perceived inaccuracies in ROPP, there is no cause to amend them.

8. Summary and conclusions

This report has reviewed the geodesy calculations in ROPP as a result of concerns raised by the beta reviewer of ROPP2.0 in 2008. The key findings are:

- The many available expressions for surface gravity differ by no more than 30 parts per million, and often much less;

- Values of the (fictitious) effective radius differ by less than 80 parts per million;

- The resulting differences in geopotential heights, or geometric heights, are almost entirely unaffected by differences in the effective radius, and therefore differ by no more than 30 ppm;

- The physical difference between the effective radius and the radius of curvature has been discussed, and the almost constant offset between them of 21km has been explained;

- The undulation between the geoid and the ellipsoid, and its spatial variation have been examined;

- The reviewer’s comments have been addressed.

The overall conclusion is that the geodesy calculations currently available in ROPP (release 6.0) are probably good enough.

9. Acknowledgments

Useful discussions with Huw Lewis, Sean Healy, Christian Marquardt and Chris Burrows are gratefully acknowledged.

10. References

GRAS SAF, 2008, “ROPP-2 Beta Test Report”, SAF/GRAS/METO/TR/ROPP/002

Heiskanen and Moritz, 1967, Physical Geodesy, W H Freeman and Co.

Kursinski E R, Hajj G A, Leroy S S and Herman B, 2000, The GPS Radio Occultation Technique, Terrestrial, Atmospheric and Oceanic Sciences, 11, 1, pp53-114.

Lambert, 1945, The International Gravity Formula, earth.geology.yale.edu/~ajs/1945A/360.pdf

Lambert, 1948, Gravity formulas for meteorological purposes. A discussion prepared in response to a request of the International Meteorological Organisation, pamphlet available from National Meteorological Library, Exeter, UK.

Lewis, 2007, Geodesy calculations in ROPP, GRAS SAF Report 02.

Lewis, 2010, pers. comm., Addendum to Geodesy calculations in ROPP, GRAS SAF Report 02.

Li and Götze, 2001, Tutorial: Ellipsoid, geoid, gravity, geodesy and geophysics, http://www.fugro-gravmag.com/resources/Technical%20Papers/Li_Goetze_Geophysics_2001.pdf

Mahoney, 2008a, A Discussion of Various Measures of Altitude, http://mtp.jpl.nasa.gov/notes/altitude/altitude.html

Mahoney, 2008b, An Exact Expression for Normal Gravity,

http://mtp.jpl.nasa.gov/notes/altitude/Gnormal.html

NASA, 2004, http://cddis.nasa.gov/926/egm96/egm96.html

National Geospatial-Intelligence Agency 2010,

http://earth-info.nga.mil/GandG/wgs84/index.html

Rodgers, 2000, Inverse Methods for Atmospheric Sounding – Theory and Practice, World Scientific.

Smithsonian meteorological tables, 1896, http://openlibrary.org/books/OL7107344M/Smithsonian_meteorological_tables

White A A, Staniforth A and Wood N, 2008, Spheroidal coordinate systems for modelling global

Atmospheres, QJRMS, 134, 261–270.

That is, the angle between the local vertical and the equatorial plane. (As opposed to the geocentric latitude, which is the angle between the radius from the earth’s centre to the given point and the equatorial plane.)

But see Sec 6 for a discussion of the undulation.

In Principia, Newton derived f=5m/4 for a uniformly rotating homogeneous blob of fluid (reported in White et al, 2008). The non-uniform mass density of the earth reconciles this with f ≈ m.