ROPP THINNER IMPLEMETATION ACROSS CRITICAL LAT/LON BOUNDARIES

The ropp_io code has been reviewed. Currently in ROPP, if any two adjacent longitude values in an occultation are separated by more than 300 degrees (i.e. span the +/-180 degree boundary), all negative longitudes are translated into the range 0-360 before thinning. No special treatment is made for latitude or azimuth values. This implementation is similar to the approach used at EUMETSAT.

A number of tests have been performed, using adjusted latitude and longitude values to simulate the thinner treatment of profiles crossing +/- 180 degrees and over-passing the pole.

Figure 1: Example lat vs lon plot from occultation atm20091122_081445_M02_2020361334_N0015. Full resolution data are plotted in black, thinned data are shown as coloured crosses. This profile was the closest found to the dateline at the time of testing. No profiles were found to cross the dateline.

Figure 2: Example DATELINE lat vs lon plot from occultation where longitude data have been offset to cross over the +/- 180 degree line.  The negative longitude points are shifted by 360 degrees prior to thinning, and the resulting thinned data match the full resolution input for all thinner implementations tested.

Figure 3: Example POLAR lat vs lon plot from occultation where latitude and longitude data have been offset to cross over the north pole and longitudes switching by 180 degrees across the pole. The data in this case are not shifted prior to thinning (since no adjacent latitude points differ by more than 300 degrees). The log-247 (and lin-247) thinner implementations fit the full resolution input data well. Implementations using the Savitsky-Golay pre-smoothing result in one outlier in the thinned data at -150 degree latitude. Note that the operational thinner used at EUMETSAT and by the GRAS SAF applies the log-247 implementation.

In summary, no erroneous data points have been identified thinning longitude data across the +/- 180 degree longitude transition with any available thinner implementation method. Further, any longitude shift caused by an occultation passing over the pole (a very rare event) is well handled by the lin-247 or log-247 thinner methods (as used in operations by EUMETSAT and the GRAS SAF). An outlying data point may result when opting to use the Savitsky-Golay pre-smoothing.

It is therefore proposed that Ticket #171 should be closed with status ‘invalid’ ahead of release ROPP-4. The ‘Known Problems’ section of the Release Notes should be updated to indicate that incorrect output may occur only for possible pole-crossing occultations and only when using Savitsky-Golay pre-smoothing.